# R2 Motor Control System

Raspberry Pi Zero W + 2 × Cytron MDD20A — 4-Motor Control

# 1. Project Overview

This project turns a Raspberry Pi Zero W into a WiFi-enabled motor controller for four brushed DC motors. Two Cytron MDD20A dual-channel H-bridge drivers provide 20A per channel (60A peak) for high-power motors. A web-based dashboard served from the Pi lets a phone or laptop control all four motors in real time over WiFi, with sub-20ms latency.

Key capabilities:

Independent speed & direction control for 4 motors

Web dashboard accessible from any device on the network

Real-time control via WebSocket (~10-15ms round trip)

Smooth PWM at 20kHz (inaudible, no motor whine)

Emergency stop (instant kill all motors)

Auto-start on boot — headless operation, no monitor or keyboard needed

# 2. Hardware Components

## 2.1 Bill of Materials

| Item | Component | Qty | Notes |
| --- | --- | --- | --- |
| 1 | Raspberry Pi Zero W | 1 | WiFi, 512MB RAM, runs Flask + pigpio |
| 2 | Cytron MDD20A | 2 | 20A/chan dual H-bridge, 6-30V supply |
| 3 | DC motors (brushed) | 4 | Up to 20A continuous each |
| 4 | Battery / PSU (6-30V) | 1 | Powers the motors via MDD20A |
| 5 | Jumper wires (female-female) | 10 | 8 signal + 2 ground for GPIO |
| 6 | MicroSD card (8GB+) | 1 | Raspbian OS installed |
| 7 | LED (any color) + 220Ω resistor | 1 | Status indicator on GPIO5 |

## 2.2 Wiring Diagram (GPIO Pin Assignment)

All connections use the bottom 10 pins of the Pi Zero W 40-pin header:

| Physical Pin | BCM GPIO | Signal | Connected To |
| --- | --- | --- | --- |
| 31 | GPIO6 | DIR — Front Left | MDD20A #1 — DIR1 |
| 32 | GPIO12 | PWM — Front Left | MDD20A #1 — PWM1 |
| 33 | GPIO13 | DIR — Front Right | MDD20A #1 — DIR2 |
| 34 | GND | Ground | MDD20A #1 — GND |
| 35 | GPIO19 | PWM — Front Right | MDD20A #1 — PWM2 |
| 36 | GPIO16 | DIR — Rear Left | MDD20A #2 — DIR1 |
| 37 | GPIO26 | PWM — Rear Left | MDD20A #2 — PWM1 |
| 38 | GPIO20 | DIR — Rear Right | MDD20A #2 — DIR2 |
| 39 | GND | Ground | MDD20A #2 — GND |
| 40 | GPIO21 | PWM — Rear Right | MDD20A #2 — PWM2 |

Important wiring rules:

Share GND between both drivers and the Pi — this is mandatory for signal reference.

Connect motor battery (6-30V) to each MDD20A's VB+/VB- terminals. The MDD20A has NO reverse-polarity protection — double-check polarity before connecting.

Power the Pi Zero W separately via its microUSB port (5V, 2.5A recommended).

All selected GPIO pins have no active alternate function conflicts on a standard Pi Zero W configuration.

## 2.3 Status LED

An LED (with a 220Ω series resistor) is connected from GPIO5 to GND. This LED indicates the system state through blink patterns controlled by the watchdog service.

| LED Pattern | System State |
| --- | --- |
| Off | No power / Pi booting |
| Slow blink (1s on, 1s off) | Stack starting — services not all active yet |
| Solid on | All services running — system ready |
| Fast blink (200ms on, 200ms off) | Service crash detected — watchdog is restarting it |
| 3 quick flashes then pause | Fatal error — watchdog cannot recover the service |

# 3. Software Architecture

## 3.1 Technology Stack

| Layer | Technology |
| --- | --- |
| PWM Generation | pigpio — DMA-based, hardware-timed PWM on any GPIO (20kHz) |
| Web Server | Flask + Flask-SocketIO — lightweight, async WebSocket support |
| Frontend | Plain HTML/CSS/JS — no framework dependencies, touch-friendly sliders |
| System Service | systemd — auto-starts pigpiod + web server on boot |

## 3.2 Files on the Pi

| File | Purpose | Key Details |
| --- | --- | --- |
| motor_control.py | Hardware abstraction layer | Connects to pigpiod, set_speed(motor, -100..100), stop_all(), 20kHz PWM |
| app.py | Flask + SocketIO web server | Binds 0.0.0.0:5000, prints IP on startup, handles WebSocket events |
| templates/index.html | Web dashboard UI | 4 touch sliders (-100 to +100), STOP button, responsive layout |
| watchdog.py | Service monitor + LED controller | Polls service states every 2s, controls GPIO5 LED patterns, restarts failed services |
| motor_control.service | systemd unit for web server | After=pigpiod.service, Restart=on-failure |
| watchdog.service | systemd unit for watchdog | After=motor_control.service pigpiod.service, Requires=motor_control.service pigpiod.service |

## 3.3 Control Flow

1. User drags a slider in the phone browser
2. JavaScript throttles to 30 updates/sec, sends via WebSocket
3. Flask-SocketIO receives the event ('set_speed')
4. app.py calls motor_control.set_speed(motor_id, value)
5. motor_control.py converts speed to DIR (high/low) + PWM duty cycle (0-1000)
6. pigpio writes to GPIO registers via /dev/gpiomem (DMA, sub-1ms)
7. MDD20A drives the motor at the commanded speed

Total latency: ~10-15ms round trip — perceived as instant by the user.

# 4. Features

## Independent 4-Motor Control

Each of the four motors has its own slider (-100 to +100). Negative values run the motor in reverse, positive values forward. Motors can be controlled independently or in any combination.

## Real-Time Web Dashboard

The web interface uses WebSockets for bidirectional communication — no page refreshes. Slider movements are reflected at the motor within ~15ms. The dashboard works on any modern browser (Chrome, Safari, Firefox).

## Emergency Stop

A prominent STOP button kills all motor outputs immediately by setting all PWM duty cycles to zero. This is a software emergency stop — for physical safety, consider adding a hardware kill switch inline with the motor battery.

## 20kHz Silent PWM

The PWM frequency is set to 20kHz, above the range of human hearing. Motors run silently without the whining noise typical of lower-frequency PWM (e.g., 1kHz from RPi.GPIO software PWM).

## Headless Auto-Start

On power-up, the Pi automatically starts the pigpio daemon and the Flask web server. No monitor, keyboard, or manual login required — just power on and connect.

## IP Announcement on Startup

The web server prints the Pi's IP address to the console on startup. When accessed via serial or SSH, the user immediately sees which IP to connect to.

## Touch-Friendly Interface

The dashboard is designed for phone use: large sliders with thick touch targets, no tiny buttons, responsive layout that fits mobile screens.

## Modular Codebase

motor_control.py is a standalone class that can be imported and used from other Python scripts, not just the web server. This allows future expansion (e.g., adding a joystick, autonomous mode, or ROS integration).

# 5. Use Cases

## Remote-Controlled Robot Vehicle

Control a 4-wheel or tracked robot from your phone. The differential-drive layout (left stick = left motors, right stick = right motors) enables intuitive driving: forward/backward/turn-in-place. The Pi Zero W mounts on the robot chassis.

## Multi-Motor Test Bench

Test and characterize multiple motors individually or simultaneously. Set different speeds on each motor to compare performance, measure current draw, or run burn-in tests.

## Wireless Conveyor / Material Handling

Control a 4-motor conveyor system from a safe distance. Individual motor control allows differential belt speeds for sorting or positioning.

## Educational Platform

Learn motor control, PWM, web services, and embedded Linux all in one project. The code is well-documented and modular, making it suitable for teaching robotics or IoT concepts.

## Prototype Development

Use as a rapid prototyping platform for motor control algorithms. The WebSocket interface makes it easy to write custom control UIs or integrate with higher-level systems like ROS.

# 6. Installation & Setup

## 6.1 Prerequisites

Raspbian OS (or Raspberry Pi OS) installed and running on the Pi Zero W.

Pi connected to the internet (for package installation).

SSH access to the Pi (username/password provided).

## 6.2 Step-by-Step

1. Install system packages: sudo apt update && sudo apt install -y pigpio python3-pip

2. Enable pigpio daemon: sudo systemctl enable pigpiod && sudo systemctl start pigpiod

3. Install Python packages: pip3 install flask flask-socketio

4. Wire the Pi to the MDD20A drivers per the wiring table in Section 2.2

5. Connect the status LED (with 220Ω resistor) from GPIO5 to GND

6. Connect motors to the MDD20A output terminals

7. Connect motor battery (6-30V) to MDD20A VB+/VB-

8. Power the Pi Zero W via microUSB

9. Copy motor_control.py, app.py, watchdog.py, and templates/index.html to /home/r2tele/motor_control/

10. Create and enable systemd services:
  sudo cp motor_control.service /etc/systemd/system/
  sudo cp watchdog.service /etc/systemd/system/
  sudo systemctl daemon-reload
  sudo systemctl enable motor_control.service watchdog.service

11. Reboot: sudo reboot

## 6.3 Connecting from a Phone

With the Pi connected to your phone's hotspot:

Open a browser on your phone (Chrome recommended)

Navigate to http://<pi-ip>:5000

Use an IP scanner app (e.g., Fing) to find the Pi's IP if unknown

With the Pi creating its own WiFi access point:

Connect your phone to the Pi's WiFi network

Open http://192.168.4.1:5000 in the browser

# 7. Safety Notes

The MDD20A has NO reverse-polarity protection on the motor power input. Connecting the battery backwards will instantly destroy the driver.

High-current motors can draw >20A — use appropriately thick wires (minimum 12 AWG recommended for the battery-to-driver connection).

The software emergency stop stops PWM signals, but the motors may coast. For mission-critical safety, add a physical relay or contactor that cuts motor battery power.

The Pi Zero W GPIO pins are 3.3V and not 5V tolerant. The MDD20A accepts 1.8-12V logic, so this is compatible. Do not connect 5V logic outputs to the Pi GPIOs.

Heat: at 20A continuous, the MDD20A may generate significant heat. Ensure adequate ventilation. The MDD20A uses NMOS H-bridges and typically doesn't require heatsinks at rated currents.

# 8. Watchdog & LED Status System

## 8.1 Overview

The watchdog service runs continuously in the background to monitor the health of all critical services (pigpiod, motor_control web server). It uses the GPIO5 LED to visually indicate the system state at a glance. If a service crashes, the watchdog automatically restarts it and signals the fault through the LED.

## 8.2 Service Dependency Chain

Power-on → pigpiod starts (systemd)
        → motor_control (app.py) starts after pigpiod
        → watchdog starts after both are active
        → watchdog confirms all alive → GPIO5 solid on

If any service crashes:
  • systemd Restart=on-failure attempts restart (primary mechanism)
  • watchdog detects the failure within 2 seconds (secondary/backup)
  • watchdog runs systemctl restart if systemd did not recover it
  • LED switches to fast blink during recovery
  • After 3 failed restart attempts, LED shows fatal error pattern

## 8.3 Watchdog Logic

1. On start: GPIO5 slow blink — system booting
2. Poll all required services every 2 seconds
3. All active → GPIO5 solid on (ready)
4. Any service down → GPIO5 fast blink + systemctl restart
5. Re-check in 5 seconds
6. Restored → back to solid on
7. Still down after 3 retries → 3-flash fatal pattern
8. Watchdog itself restarts automatically via systemd if it crashes

## 8.4 Edge Cases Handled

Watchdog crashes → systemd restarts it (Restart=on-failure in watchdog.service)

pigpiod frozen (not crashed) → systemd watchdog timeout kills and restarts it

Race condition on boot → motor_control waits for pigpiod socket before connecting

Network down but motors still work → watchdog only checks services, not network

Manual service restart → watchdog detects it came back and restores solid LED

# 9. Future Enhancements

Differential drive mode: add a tank-style control layout (left stick = left motors, right stick = right motors)

Battery voltage monitoring via MDD20A or external ADC

Speed feedback from encoders for closed-loop PID control

WiFi AP mode: Pi creates its own network so your phone connects directly without a hotspot

MQTT integration for IoT / home automation connectivity

ROS 2 integration for advanced robotics applications

# 10. Project Structure & Test Suite

## 10.1 File Tree

motor_control/
├── motor_control.py          # Hardware abstraction — pigpio DMA PWM, 4 motors
├── app.py                    # Flask + SocketIO web server
├── watchdog.py               # Service monitor + GPIO5 LED controller
├── motor_control.service     # systemd unit for web server
├── watchdog.service          # systemd unit for watchdog
├── deploy.sh                 # One-shot deploy script
├── templates/
│   └── index.html            # Web dashboard (4 sliders + STOP)
└── tests/
    ├── __init__.py
    ├── mock_pigpio.py        # Mock pigpio library for testing
    ├── test_motor_control.py  # 29 tests — init, speed, direction, clamping, cleanup
    ├── test_app.py            # 7 tests — HTTP routes, WebSocket events
    └── test_watchdog.py       # 14 tests — LED patterns, service detection, restart

## 10.2 Test Coverage Summary

| Test File | Tests | What It Covers |  |
| --- | --- | --- | --- |
| test_motor_control.py | 29 | Init, set_speed (±100), clamping, set_all, stop, cleanup, error handling, independent channels |  |
| test_app.py | 7 | HTTP 200, WebSocket set_speed/set_speeds/stop, invalid motor ignored, connect sends state |  |
| test_watchdog.py | 14 | LED on/off/blink patterns, service_active detection, restart call, wait_for_services timeout |  |

All 50 tests pass without hardware — they use mock_pigpio.py which simulates the pigpio library. Tests can be run on any machine (no Pi required) with:
  cd motor_control && pip install pytest && python3 -m pytest tests/ -v